INTERSTATE PRODUCTS, INC.

3921 Sawyer Road

Sarasota, FL  34233

800-IPI-SAY HI     www.interstateproducts.com    fax 800-44-TO-FAX

MATERIAL SAFETY DATA

1.)  PRODUCT INFORMATION

Date     September 2006

Trade Name    TZ-61-50M Green Fire Ice Melt Pellets

Manufactured For    Interstate Products, Inc.,

CAS Registry No.     See below for individual ingredients

EMERGENCY TELEPHONE   800-424-9300

Telephone For Information:  800-474-7294

2.)  HAZARDOUS INGREDIENTS

NTP / IARC								TLV						VAPOR

INGREDIENT			      OSHA CARC		PERCENT		PPM    MG/m3		LEL	   PRESSURE

Magnesium Chloride					No					34				 	Not Established

CAS# 10043524

3.)  PHYSICAL DATA

Boiling Point:  212 Degrees F

Percent Volatile (By Volume):  76

Weight Per Gallon:  10.6 Lbs.

Vapor Density:     Heavier       XX                 Lighter                  Than Air

Evaporation Rate:  Faster					  Slower   XX		Than Ether

4.)  FIRE AND EXPLOSION HAZARD DATA

Flash Point (method used):  NA							Special Fire Fighting Procedures:  NA

Flammable Limits:  NA		LEL:  NA						Unusual Fire and Explosion Hazards:  NA

Extinguishing Media:  NA									DOT Category:  Non-Flammable

5.)  REACTIVITY DATA

Stability:  Stable

Incompatibility (materials to avoid):  NA

Hazardous Decomposition or Byproducts:  May form toxic materials-chloride and calcium oxide.

Hazardous Polymerization:  Cannot occur

6.)  HEALTH HAZARD DATA

Effects of Overexposure:  Eyes:  May cause irritation, redness, tearing, and blurred vision.  Skin:  May cause moderate irritation, defatting, and dermatitis.  Inhalation:  No health hazard presented..  Ingestion:  May cause gastrointestinal irritation.

First Aid:  Eyes:  Flush with large amounts of water, lifting upper and lower lids occasionally.  Seek

medical attention.  Skin:  Thoroughly wash exposed area with soap and water.  Remove contaminated clothing.  Launder contaminated clothing before re-use.  Ingestion:  Do not induce vomiting, keep person warm and quiet.  Seek medical attention.

7.)  SAFE HANDLING AND USE

Material Released or Spilled:  Absorb liquid on paper, vermiculite, floor absorbent, or other absorbent material.

Waste Disposal Method:  Dispose of in accordance with local, state, and federal regulations.

Special Precautions:  Containers of this material may be hazardous when emptied.  Since emptied containers retain product residues (vapor, liquid, and/or solid), all hazard precautions given in this data sheet must be observed.  KEEP OUT OF REACH OF CHILDREN.

8.)  CONTROL MEASURES

Respiratory Protection:  NA

Ventilation:  NA

Protective Gloves:  Wear resistant gloves such as Buna-N.

Eye Protection:  Chemical splash goggles in compliance with OSHA regulations are advised; however, OSHA regulations also permit other types of safety glasses.   (See your safety equipment supplier).)

Other Protective Clothing or Equipment:  To prevent repeated or prolonged skin contact wear impervious clothing and boots.

NOTICE:  The information contained on this Material Safety Data Sheet is considered accurate as of the date of publication.  It is not necessarily all inclusive nor fully adequate in every circumstance.  The suggestions should not be confused with, nor followed in violation of applicable laws, regulations, rules or insurance requirements.  No warranty, express or implied, of merchantability, fitness, accuracy of data, or the results to be obtained from the use thereof is made.  The supplier assumes no responsibility for injury or damages resulting from the inappropriate use of this product.